IR MODULE

The transmitter part of the sensor project is an Infrared (IR) Led which transmits continuous IR rays to be received by an IR receiver. The output of the receiver varies depending upon its reception of IR rays. Since this variation cannot be analyzed as such, therefore this output can be fed to a comparator. Here operational amplifier (op-amp) of LM 339 is used as comparator.

When the IR receiver does not receive signal the potential at the inverting input goes higher than that that at non-inverting input of the comparator (LM 339). Thus the output of the comparator goes low and the LED does not glow .When the IR receiver receives signal the potential at the inverting input goes low. Thus the output of the comparator (LM 339) goes high and the LED starts glowing. Resistor R1 (100), R2 (10k) and R3 (330 ) are used to ensure that minimum 10 mA current passes through the IR LED, photodiode and normal LED, respectively. Resistor VR2 (preset=5k) is used to adjust the output. Resistor VR1 (preset=10k) is used to set the sensitivity of the circuit. Read more about IR sensor here.

IC4033

The functions of CD4033B IC pinouts:

Pin#1: It’s the clock input pinout of the IC, which is assigned for accepting positive clock signals or the pulses which needs to be checked or counted.

Pin#2: It’s the clock inhibit pinout of the IC, As the name refers to, this pinout could be used for inhibiting the IC from responding to the input pulses by configuring this pinout to the positive supply or the Vdd. Conversely in order to allow normal functioning of the IC this pinout should be grounded.

Pin#3/#4: These are the Ripple blanking IN and Ripple blanking OUT pinouts of the IC, which provides the user with the option of either allowing the non-significant zeros to be displayed or to be left out from the connected digital displays.

Pin#14: It’s the “lamp test” pinout of the IC. As the name signifies it is used for testing the connected digital displays in terms of illumination level. When this pinout is connected to a high level or the positive supply, the normal function of the IC is disabled and all the digits of the 7 segment display are applied with a high state so that the digits are allowed to get illuminated together. This allows us to test the intensity levels of the digits and if any of the display digits are not functioning optimally or are dim due to some malfunction.

Pin#6,7,9,10,11,12,13: All these pinouts are the outputs of the IC which are configured with the discussed 7 segment digital display module.

Pin#15: It’s the reset input of the IC, a high logic or applying the supply voltage to this pin resets the IC completely, resulting in clearing all the data from the display and restoring it to zero.

Pin#5: It’s the carryout pinout of the IC, it sends a high logic output after every 10 legit clocks at the clock pin#1 of the IC. Thus pin#5 is used as a clock output or a carry forward extension for the next corresponding IC 4033 when many of these are cascaded together in a multi-digit display counter systems.

Pin#16 is the Vdd or the supply input of the IC.

Pin#8 is the Vss, or the ground or the negative supply input pinout of the IC 4033.

The IC works best with supply voltages between 5V and 20V.

#include<reg51.h>

#define on 0

#define off 1

#define reset 1

int scan=1;

//for first segment

int count_1=0;

int runtime_1 = 30;

sbit R1 = P2^0;

sbit Y1 = P2^1;

sbit G1 = P2^2;

sbit SS_CLK_1 = P2^3;

sbit SS_RST_1 = P2^4;

sbit module1_1 = P0^0;

//for second segment

int count_2=0;

int runtime_2 = 30;

sbit R2 = P3^0;

sbit Y2 = P3^1;

sbit G2 = P3^2;

sbit SS_CLK_2 = P3^4;

sbit SS_RST_2 = P3^3;

sbit module2_1 = P0^1;

// for third segment

int count_3=0;

int runtime_3 = 30;

sbit R3 = P2^5;

sbit Y3 = P2^6;

sbit G3 = P2^7;

sbit SS_CLK_3 = P3^7;

sbit SS_RST_3 = P3^6;

sbit module3_1 = P0^2;

// for fourth segment

int count_4=0;

int rntime_4 = 30;

sbit R4 = P1^0;

sbit Y4 = P1^1;

sbit G4 = P1^2;

sbit SS_CLK_4 = P1^4;

sbit SS_RST_4 = P1^3;

sbit module4_1 = P0^3;

void Delay(unsigned int x)

{

unsigned int i,j;

for(i=0;i<x;i++)

}

void _reset(int d)

{

int delay = 30;

switch(d)

{

case 0: // reset all

SS_RST_1=1;

SS_RST_2=1;

SS_RST_3=1;

SS_RST_4=1;

Delay(delay);

SS_RST_1=0;

SS_RST_2=0;

SS_RST_3=0;

SS_RST_4=0;

Delay(delay);

count_1=0;

count_2=0;

count_3=0;

count_4=0;

break;

case 1:

SS_RST_1=1;

msDelay(delay);

SS_RST_1=0;

msDelay(delay);

break;

case 2:

SS_RST_2=1;

msDelay(delay);

SS_RST_2=0;

msDelay(delay);

break;

case 3:

SS_RST_3=1;

msDelay(delay);

SS_RST_3=0;

msDelay(delay);

break;

case 4:

SS_RST_4=1;

msDelay(delay);

SS_RST_4=0;

msDelay(delay);

break;

}

}

void clock(int d)

{

int delay = 20;

switch(d)

{

case 1:

count_1++;

SS_CLK_1=1;

msDelay(delay);

SS_CLK_1=0;

msDelay(delay);

break;

case 2:

count_2++;

SS_CLK_2=1;

msDelay(delay);

SS_CLK_2=0;

msDelay(delay);

break;

case 3:

count_3++;

SS_CLK_3=1;

Delay(delay);

SS_CLK_3=0;

Delay(delay);

break;

case 4:

count_4++;

SS_CLK_4=1;

Delay(delay);

SS_CLK_4=0;

Delay(delay);

break;

}

}

void main()

{

P2=0x00;

P1=0x00;

P0=0xFF;

R1=off;

Y1=off;

G1=off;

R2=off;

Y2=off;

G2=off;

R3=off;

Y3=off;

G3=off;

R4=off;

Y4=off;

G4=off;

R1=on;

R2=on;

R3=on;

R4=on;

ss_reset(0);

while(1)

{

clock(1);

clock(2);

clock(3);

clock(4);

switch(scan)

{

case 1:

// scan first segment

if(count_1 <= 3)

{

if( module1_1 == 1)

{

runtime_1=60;

}

else

{

runtime_1=30;

}

}

R1=off;

G1=on;

if(count_1 >= runtime_1)

{

G1=off;

Y1=on;

msDelay(3000);

Y1=off;

ss_reset(0);

scan=2;

R1=on;

}

break;

case 2:

// scan second segment

if(count_2 <= 3)

{

if( module2_1 == 1)

{

runtime_2=60;

}

else

{

runtime_2=30;

}

}

R2=off;

G2=on;

if(count_2 >= runtime_2)

{

G2=off;

Y2=on;

msDelay(3000);

Y2=off;

ss_reset(0);

scan=3;

R2=on;

}

break;

case 3:

// scan third segment

if(count_3 <= 3)

{

if( module3_1 == 1)

{

runtime_3=60;

}

else

{

runtime_3=30;

}

}

R3=off;

G3=on;

if(count_3 >= runtime_3)

{

G3=off;

Y3=on;

msDelay(3000);

Y3=off;

ss_reset(0);

scan=4;

R3=on;

}

break;

case 4:

// scan fourth segment

if(count_4 <= 3)

{

if( module4_1 == 1)

{

runtime_4=60;

}

else

{

runtime_4=30;

}

}

R4=off;

G4=on;

if(count_4 >= runtime_4)

{

G4=off;

Y4=on;

msDelay(3000);

Y4=off;

ss_reset(0);

scan=1;

R4=on;

}

break;

}

}

}

Part                                                     Value                                                     Quantity

DIS1                                               COMMON CATHODE 	8

IC2                                                            4033                               	8

R1                                                             33K            	8

R2                                                            47K            	4

R3                                                              470E           	4

module                                                         IR MODULE      	8

BRIDGE                   	rectifier	1

C1                                                                 33pf        	2

C2                                                                  470uf       	1

C3                                                            .01uf       	2

CRYSTAL                                               11.0592 mhz 	1

IC1                                                               AT89S51                           	1

IC2                                                               7805TV      	1

JP1                                                        TRANSFORMER         	1

LED                                                               5mm                  	1

R1                                                                       10k         	1

R2                                                                           560E       	1

RN10                                                                           10 k        	1

Switch1                                                                     on/off          	1

Switch2                                                                     push switch 	1