DISPLAY

LCD (Liquid Crystal Display) screen is an electronic display module and find a wide range of applications. A 16x2 LCD display is very basic module and is very commonly used in various devices and circuits. These modules are preferred over seven segments and other multi segment LEDs. The reasons being: LCDs are economical; easily programmable; have no limitation of displaying special & even custom characters (unlike in seven segments), animations and so on.

A 16x2 LCD means it can display 16 characters per line and there are 2 such lines. In this LCD each character is displayed in 5x7 pixel matrix. This LCD has two registers, namely, Command and Data.

The command register stores the command instructions given to the LCD. A command is an instruction given to LCD to do a predefined task like initializing it, clearing its screen, setting the cursor position, controlling display etc. The data register stores the data to be displayed on the LCD. The data is the ASCII value of the character to be displayed on the LCD. Click to learn more about internal structure of a LCD.

### Pin Diagram:

IR MODULE

The transmitter part of the sensor project is an Infrared (IR) Led which transmits continuous IR rays to be received by an IR receiver. The output of the receiver varies depending upon its reception of IR rays. Since this variation cannot be analyzed as such, therefore this output can be fed to a comparator. Here operational amplifier (op-amp) of LM 339 is used as comparator.

When the IR receiver does not receive signal the potential at the inverting input goes higher than that that at non-inverting input of the comparator (LM 339). Thus the output of the comparator goes low and the LED does not glow .When the IR receiver receives signal the potential at the inverting input goes low. Thus the output of the comparator (LM 339) goes high and the LED starts glowing. Resistor R1 (100), R2 (10k) and R3 (330 ) are used to ensure that minimum 10 mA current passes through the IR LED, photodiode and normal LED, respectively. Resistor VR2 (preset=5k) is used to adjust the output. Resistor VR1 (preset=10k) is used to set the sensitivity of the circuit. Read more about IR sensor here.

#include<reg52.h>							 //including sfr registers for ports of the controller

#include<lcd.h>

#define on 0

#define off 1

int scan=1;

//LCD Module Connections

sbit RS = P0^0;

sbit EN = P0^1;

sbit D0 = P2^0;

sbit D1 = P2^1;

sbit D2 = P2^2;

sbit D3 = P2^3;

sbit D4 = P2^4;

sbit D5 = P2^5;

sbit D6 = P2^6;

sbit D7 = P2^7;

//for first segment

int count_1=0;

int runtime_1 = 15;

sbit R1 = P1^7;

sbit G1 = P1^6;

sbit module1_1 = P3^6;

sbit module1_2 = P3^7;

//for second segment

int count_2=0;

int runtime_2 = 15;

sbit R2 = P1^5;

sbit G2 = P1^4;

sbit module2_1 = P3^5;

sbit module2_2 = P3^4;

//for third segment

int count_3=0;

int runtime_3 = 15;

sbit R3 = P1^3;

sbit G3 = P1^2;

sbit module3_1 = P3^1;

sbit module3_2 = P3^0;

//for fourth segment

int count_4=0;

int runtime_4 = 15;

sbit R4 = P1^0;

sbit G4 = P1^1;

sbit module4_1 = P3^2;

sbit module4_2 = P3^3;

//

//End LCD Module Connections

void Delay(int a)

{

int j;

int i;

for(i=0;i<a;i++)

{

for(j=0;j<100;j++)

{

}

}

}

char toChar(int a)

{

switch(a)

{

case 1:

return '1';

break;

case 2:

return '2';

break;

case 3:

return '3';

break;

case 4:

return '4';

break;

case 5:

return '5';

break;

case 6:

return '6';

break;

case 7:

return '7';

break;

case 8:

return '8';

break;

case 9:

return '9';

break;

case 0:

return '0';

break;

}

}

void main()

{

int i,j,k,l;

char c;

Lcd8_init();

Lcd8_Set_Cursor(1,1);

Lcd8_Write_String("Traffic Light");

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Control");

R1=off;

G1=off;

R2=off;

G2=off;

Delay(100);

R3=off;

G3=off;

R4=off;

G4=off;

Delay(100);

R1=on;

R2=on;

R3=on;

R4=on;

Delay(100);

G1=off;

G2=off;

G3=off;

G4=off;

Delay(100);

while(1)

{

switch(scan)

{

case 1:

// scan first segment

if(count_1 <= 0)

{

runtime_1=15;

if( module1_1 == 1)

{

runtime_1=30;

}

if(module1_2 == 1)

{

runtime_1=60;

}

count_1=runtime_1;

}

R1=off;

G1=on;

count_1--;

Lcd8_Clear();

i=count_1;

j=i%10;

k=i/10;

l=i/100;

k=k-l*10;

Lcd8_Set_Cursor(1,3);

c=toChar(j);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(1,2);

c=toChar(k);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(2,1);

c=toChar(l);

Lcd8_Write_Char(c);

if(runtime_1 == 60)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg1:60 Secs");

}

else if(runtime_1 == 30)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg1:30 Secs");

}

else

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg1:15 Secs");

}

if(count_1 == 0)

{

G1=off;

count_1=0;

scan=2;

R1=on;

}

break;

case 2:

// scan second segment

if(count_2 <= 0)

{

runtime_2=15;

if( module2_1 == 1)

{

runtime_2=30;

}

if(module2_2 == 1)

{

runtime_2=60;

}

count_2=runtime_2;

}

R2=off;

G2=on;

count_2--;

Lcd8_Clear();

i=count_2;

j=i%10;

k=i/10;

l=i/100;

k=k-l*10;

Lcd8_Set_Cursor(1,3);

c=toChar(j);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(1,2);

c=toChar(k);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(2,1);

c=toChar(l);

Lcd8_Write_Char(c);

if(runtime_2 == 60)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg2:60 Secs");

}

else if(runtime_2 == 30)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg2:30 Secs");

}

else

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg2:15 Secs");

}

if(count_2 == 0)

{

G2=off;

count_2=0;

scan=3;

R2=on;

}

break;

case 3:

// scan third segment

if(count_3 <= 0)

{

runtime_3=15;

if( module3_1 == 1)

{

runtime_3=30;

}

if(module3_2 == 1)

{

runtime_3=60;

}

count_3=runtime_3;

}

R3=off;

G3=on;

count_3--;

Lcd8_Clear();

i=count_3;

j=i%10;

k=i/10;

l=i/100;

k=k-l*10;

Lcd8_Set_Cursor(1,3);

c=toChar(j);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(1,2);

c=toChar(k);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(2,1);

c=toChar(l);

Lcd8_Write_Char(c);

if(runtime_3 == 60)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg3:60 Secs");

}

else if(runtime_3 == 30)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg3:30 Secs");

}

else

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg3:15 Secs");

}

if(count_3 == 0)

{

G3=off;

count_3=0;

scan=4;

R3=on;

}

break;

case 4:

// scan fourth segment

if(count_4 <= 0)

{

runtime_4=15;

if( module4_1 == 1)

{

runtime_4=30;

}

if(module4_2 == 1)

{

runtime_4=60;

}

count_4=runtime_4;

}

R4=off;

G4=on;

count_4--;

Lcd8_Clear();

i=count_4;

j=i%10;

l=i/100;

k=k-l*10;

c=toChar(j);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(1,2);

c=toChar(k);

Lcd8_Write_Char(c);

Lcd8_Set_Cursor(2,1);

c=toChar(l);

Lcd8_Write_Char(c);

if(runtime_4 == 60)

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg4:60 Secs");

}

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg4:30 Secs");

}

else

{

Lcd8_Set_Cursor(2,1);

Lcd8_Write_String("Seg4:15 Secs");

}

if(count_4 == 0)

{

G4=off;

count_4=0;

scan=1;

R4=on;

}

break;

}

Delay(700);

}

}

Part                                                     Value                                                     Quantity

LCD                                                              16*2 	1

VR                                                                 20K            	1

module                                                         IR MODULE      	8

BRIDGE                   	rectifier	1

C1                                                                 33pf        	2

C2                                                                  470uf       	1

C3                                                            .01uf       	2

CRYSTAL                                               11.0592 mhz 	1

IC1                                                               AT89S51                           	1

IC2                                                               7805TV      	1

JP1                                                        TRANSFORMER         	1

LED                                                               5mm                  	1

R1                                                                       10k         	1

R2                                                                           560E       	1

RN10                                                                           10 k        	1

Switch1                                                                     on/off          	1

Switch2                                                                     push switch 	1